1. Enable Detailed Logging

Issue:

Your logging level is set to ERROR, which hides useful debug information.

Solution:

Change the logging level to DEBUG to capture detailed logs that can help identify where the process is failing.

python

Copy code

# ================================

# 1. Configure Logging

# ================================

logging.basicConfig(

level=logging.DEBUG,  # Changed from ERROR to DEBUG for detailed logs

format='%(asctime)s [%(levelname)s] %(message)s',

handlers=[logging.StreamHandler(sys.stdout)]

)

Benefit: This allows you to see detailed information about each step, making it easier to pinpoint where the process is failing.

2. Verify Reed-Solomon (RS) Encoding and Decoding

Issue:

RS decoding fails, indicating possible issues with RS parameters or bitstream alignment.

Solution:

Ensure that RS encoding and decoding are correctly implemented and that their parameters match.

- Check RS Parameters:

  - Encoding:

    - Packet Size (k): 188 bytes.

    - Parity Bytes (n - k): 47 bytes.

    - Total Encoded Packet Size (n): 235 bytes.

  - Decoding:

    - Packet Size (n): 235 bytes.

    - Original Data Size (k): 188 bytes.

Ensure both encoding and decoding use these parameters consistently.

- Test RS Codec Independently: Use the test_rs_codec function you provided to ensure RS encoding and decoding work correctly without introducing errors.

python

Copy code

def test_rs_codec():

"""

Test Reed-Solomon encoding and decoding with known data.

"""

try:

# Initialize Reed-Solomon Codec

rs_codec = RSCodec(47)  # RS(n=235, k=188)

# Create a sample data packet (188 bytes)

original_data = list(range(188))  # Example data: [0, 1, 2, ..., 187]

print("Original Data:", original_data)

# Encode the data

encoded_data = rs_codec.encode(original_data)

print("Encoded Data:", encoded_data)

# Introduce errors (e.g., flip bits in some bytes)

corrupted_data = encoded_data.copy()

corrupted_data[10] ^= 0xFF  # Flip all bits in byte 10

corrupted_data[50] ^= 0xAA  # Flip some bits in byte 50

print("Corrupted Data:", corrupted_data)

# Decode the corrupted data

decoded_data = rs_codec.decode(corrupted_data)

print("Decoded Data:", decoded_data)

# Verify that decoded data matches original

assert decoded_data == original_data, "Decoded data does not match original!"

print("RS Codec Test Passed: Decoded data matches original.")

except ReedSolomonError as e:

print(f"RS Codec Test Failed: {e}")

except AssertionError as ae:

print(f"RS Codec Test Failed: {ae}")

except Exception as ex:

print(f"An unexpected error occurred during RS Codec Test: {ex}")

- Run RS Codec Test: Execute the test_rs_codec() function to ensure that RS encoding and decoding work as expected. If this test passes, the RS codec is likely implemented correctly.

Expected Output:

less

Copy code

Original Data: [0, 1, 2, ..., 187]

Encoded Data: bytearray(b'\x00\x01\x02\x03\x04\x05\x06\x07\x08\t\n\x0b\x0c\r\x0e\x0f\x10\x11\x12\x13\x14\x15\x16\x17\x18\x19\x1a\x1b\x1c\x1d\x1e\x1f !"#$%&\'()*+,-./0123456789:;<=>?@ABCDEFGHIJKLMNOPQRSTUVWXYZ[\\]^_`abcdefghijklmnopqrstuvwxyz{|}~\x7f\x80\x81\x82\x83\x84\x85\x86\x87\x88\x89\x8a\x8b\x8c\x8d\x8e\x8f\x90\x91\x92\x93\x94\x95\x96\x97\x98\x99\x9a\x9b\x9c\x9d\x9e\x9f\xa0\xa1\xa2\xa3\xa4\xa5\xa6\xa7\xa8\xa9\xaa\xab\xac\xad\xae\xaf\xb0\xb1\xb2\xb3\xb4\xb5\xb6\xb7\xb8\xb9\xba\xbbU%ox\xb0\xa5\x08\x80\xf1\xb6\xc7,\xd4\x9f\x1a\x83\xd92\xf4\xcc]\xadp\xc8\x19\xc4\xc1$X\x93v!\x9c\xa3\x03\x80\xc5\x90s\x18\xc7\xa3p! \xe9\xc5')

Corrupted Data: bytearray(b'\x00\x01\x02\x03\x04\x05\x06\x07\x08\t\xf5\x0b\x0c\r\x0e\x0f\x10\x11\x12\x13\x14\x15\x16\x17\x18\x19\x1a\x1b\x1c\x1d\x1e\x1f !"#$%&\'()*+,-./01\x983456789:;<=>?@ABCDEFGHIJKLMNOPQRSTUVWXYZ[\\]^_`abcdefghijklmnopqrstuvwxyz{|}~\x7f\x80\x81\x82\x83\x84\x85\x86\x87\x88\x89\x8a\x8b\x8c\x8d\x8e\x8f\x90\x91\x92\x93\x94\x95\x96\x97\x98\x99\x9a\x9b\x9c\x9d\x9e\x9f\xa0\xa1\xa2\xa3\xa4\xa5\xa6\xa7\xa8\xa9\xaa\xab\xac\xad\xae\xaf\xb0\xb1\xb2\xb3\xb4\xb5\xb6\xb7\xb8\xb9\xba\xbbU%ox\xb0\xa5\x08\x80\xf1\xb6\xc7,\xd4\x9f\x1a\x83\xd92\xf4\xcc]\xadp\xc8\x19\xc4\xc1$X\x93v!\x9c\xa3\x03\x80\xc5\x90s\x18\xc7\xa3p! \xe9\xc5')

RS Codec Test Failed: Reed-Solomon decoding failed.

Interpretation:

  - Without Errors: The RS Codec Test Passed: Decoded data matches original.

  - With Errors: As shown, introducing errors causes RS decoding to fail, which is expected behavior.

Conclusion: If test_rs_codec passes when not introducing errors and fails when introducing errors, RS encoding and decoding are likely implemented correctly.

3. Simplify the Test Pipeline

Issue:

Complex pipelines make it difficult to identify where errors are introduced.

Solution:

Simplify your test_pipeline to isolate and identify where the corruption occurs. Start by removing or temporarily disabling certain steps to see if RS decoding works without them.

Steps:

- Remove Scrambling and Descrambling: Scrambling introduces randomness, which complicates debugging. Temporarily disable these steps to see if RS decoding works without them.

python

Copy code

# In test_pipeline, comment out scrambling and descrambling

# scrambled = scramble_bitstream(rs_encoded)

scrambled = rs_encoded.copy()  # No scrambling

...

# descrambled_bits = descramble_bitstream(deinterleaved_bits)

descrambled_bits = deinterleaved_bits.copy()  # No descrambling

- Remove Interleaving and Deinterleaving: Similarly, interleaving rearranges bits, which can obscure where the issue is. Temporarily disable these steps.

python

Copy code

# In test_pipeline, comment out interleaving and deinterleaving

# interleaved = interleave_data(scrambled)

interleaved = scrambled.copy()  # No interleaving

...

# deinterleaved_bits = deinterleave_data(decoded_bits)

deinterleaved_bits = decoded_bits.copy()  # No deinterleaving

- Run the Simplified Pipeline: Modify the test_pipeline function to exclude scrambling, interleaving, and their inverse operations.

python

Copy code

def test_pipeline_simplified():

"""

Simplified Test Pipeline without scrambling and interleaving.

"""

try:

logging.info("Starting Simplified Test Pipeline.")

# Step 1: Create a controlled bitstream (188 bytes = 1504 bits)

test_bits = np.random.randint(0, 2, 188 * 8, dtype=np.uint8)

logging.debug(f"Test Pipeline: Created bitstream with {len(test_bits)} bits.")

# Step 2: RS Encode

rs_encoded = rs_encode(test_bits)

assert rs_encoded is not None, "RS encoding failed."

logging.debug(f"Test Pipeline: RS encoding completed. Encoded bitstream length: {len(rs_encoded)} bits.")

# Step 3: No Scramble

scrambled = rs_encoded.copy()

logging.debug("Test Pipeline: Skipped scrambling.")

# Step 4: No Interleave

interleaved = scrambled.copy()

logging.debug("Test Pipeline: Skipped interleaving.")

# Step 5: Convolutional Encoding

conv_encoded = trellis_encoder(interleaved)

logging.debug(f"Test Pipeline: Convolutional encoding completed. Length: {len(conv_encoded)} bits.")

# Step 6: Pad for 8-VSB (make multiple of 3 bits)

padding_length = (3 - (len(conv_encoded) % 3)) % 3

if padding_length > 0:

conv_encoded = np.pad(conv_encoded, (0, padding_length), 'constant')

logging.debug(f"Test Pipeline: Padded convolutionally encoded bits with {padding_length} zeros.")

# Step 7: Map to 8-VSB symbols

expected_symbols = len(conv_encoded) // 3

symbols = map_bits_to_amplitude_8vsb(conv_encoded, expected_symbols)

logging.debug(f"Test Pipeline: Mapped to {len(symbols)} 8-VSB symbols.")

# Step 8: Insert Pilot

symbols_with_pilot = insert_pilot_sequence(symbols, pilot_length=2)  # Use smaller pilot for test

logging.debug(f"Test Pipeline: Inserted pilot sequence. Total symbols: {len(symbols_with_pilot)}.")

# Step 9: Modulate

modulated = generate_modulated_signal(symbols_with_pilot, CARRIER_FREQ, SAMPLING_RATE)

logging.debug(f"Test Pipeline: Generated modulated signal with {len(modulated)} samples.")

# Step 10: Demodulate

demodulated = demodulate_8vsb(modulated, CARRIER_FREQ, SAMPLING_RATE, LOWPASS_FILTER)

logging.debug(f"Test Pipeline: Demodulated signal length: {len(demodulated)}.")

# Step 11: Timing Recovery

timing_corrected = timing_recovery(demodulated, SPS, expected_length=len(symbols_with_pilot))

logging.debug(f"Test Pipeline: Timing corrected signal length: {len(timing_corrected)}.")

# Step 12: Quantize Symbols

quantized = quantize_symbols(timing_corrected, VSB_LEVELS)

logging.debug(f"Test Pipeline: Quantized symbols: {quantized[:10]}.")

# Step 13: Remove Pilot

symbols_no_pilot = quantized[2:]  # Remove first 2 pilot symbols

logging.debug(f"Test Pipeline: Symbols without pilot: {symbols_no_pilot[:10]}.")

# Step 14: Map to Bits

recovered_bits = map_symbols_to_bits_8vsb(symbols_no_pilot)

logging.debug(f"Test Pipeline: Recovered bits length: {len(recovered_bits)}.")

# Step 15: Viterbi Decoding

decoded_bits = viterbi_decoder(recovered_bits)

assert decoded_bits is not None, "Viterbi decoding failed."

logging.debug(f"Test Pipeline: Viterbi decoding completed. Decoded bits length: {len(decoded_bits)}.")

# Step 16: No Deinterleaving

deinterleaved_bits = decoded_bits.copy()

logging.debug("Test Pipeline: Skipped deinterleaving.")

# Step 17: No Descramble

descrambled_bits = deinterleaved_bits.copy()

logging.debug("Test Pipeline: Skipped descrambling.")

# Step 18: RS Decode

rs_decoded = rs_decode(descrambled_bits)

assert rs_decoded is not None, "RS decoding failed."

logging.debug(f"Test Pipeline: RS decoding completed. Decoded bits length: {len(rs_decoded)}.")

# Step 19: Compare Original and Decoded Bits

rs_decoded_trimmed = rs_decoded[:len(test_bits)]

if np.array_equal(test_bits, rs_decoded_trimmed):

logging.info("Simplified Test Pipeline Passed: Decoded bitstream matches original.")

print("Simplified Test Pipeline Passed: Decoded bitstream matches original.")

else:

logging.error("Simplified Test Pipeline Failed: Decoded bitstream does not match original.")

print("Simplified Test Pipeline Failed: Decoded bitstream does not match original.")

except Exception as e:

print(f"Simplified Test Pipeline Failed: {e}")

- Run the Simplified Pipeline: Execute test_pipeline_simplified() to see if RS decoding passes without scrambling and interleaving.

Expected Outcome:

  - If RS Decoding Passes: The issue lies in the scrambling and/or interleaving steps.

  - If RS Decoding Fails: The issue is elsewhere, possibly in convolutional encoding/decoding or bitstream alignment.

Benefit: This helps isolate the problematic part of the pipeline by removing complex steps that might introduce errors.

4. Verify Convolutional Encoding and Viterbi Decoding

Issue:

Potential mismatch between convolutional encoding and Viterbi decoding parameters.

Solution:

Ensure that the convolutional encoder and Viterbi decoder are correctly configured and are inverses of each other.

- Check Trellis Definition: Make sure the trellis is correctly defined with the appropriate memory and generator polynomials.

python

Copy code

def define_trellis():

def octal_to_bitlist(octal, constraint_length):

"""

Convert an octal number to a list of bits with leading zeros.

"""

binary = bin(octal)[2:].zfill(constraint_length)

bitlist = [int(bit) for bit in binary]

return bitlist

# Generator polynomials in octal

g0_octal = 0o171

g1_octal = 0o133

# Constraint length

memory = 6

constraint_length = memory + 1

memory_list = [memory]

# Convert octal to bit lists

g0_bits = octal_to_bitlist(g0_octal, constraint_length)

g1_bits = octal_to_bitlist(g1_octal, constraint_length)

# Define g_matrix

g_matrix = [

g0_bits,

g1_bits

]

# Create Trellis

trellis = Trellis(memory_list, g_matrix)

logging.debug(f"Trellis created with memory={memory} and g_matrix={g_matrix}.")

return trellis

- Ensure Consistent Encoding and Decoding: Both encoding and decoding should use the same trellis and parameters.

python

Copy code

# Encoding

def trellis_encoder(bitstream):

"""

Convolutionally encode a bitstream using the defined trellis.

"""

encoded_bits = conv_encode(bitstream.astype(np.int8), trellis)

return encoded_bits

# Decoding

def viterbi_decoder(encoded_bits):

"""

Decode the encoded bits using the Viterbi algorithm.

"""

try:

decoded_bits = commpy_viterbi_decode(

encoded_bits.astype(np.int8),

trellis,

tb_depth=35,  # Adjusted based on memory and constraint length

decoding_type='hard'  # Ensure decoding type matches encoding

)

# Validate that all decoded bits are either 0 or 1

if decoded_bits is None or not np.all(np.isin(decoded_bits, [0, 1])):

logging.error("Viterbi decoding produced invalid or NaN bit values.")

return None

return decoded_bits

except Exception as e:

logging.error(f"Exception during Viterbi decoding: {e}")

return None

- Test Convolutional Encoding and Decoding Independently: Create a separate test to ensure that encoding and decoding work correctly.

python

Copy code

def test_conv_encoding_decoding():

"""

Test convolutional encoding and Viterbi decoding.

"""

try:

# Create a sample bitstream

original_bits = np.array([0, 1, 0, 1, 1, 0, 1, 0, 1, 1], dtype=np.int8)

logging.debug(f"Original Bits: {original_bits}")

# Convolutional Encoding

encoded_bits = trellis_encoder(original_bits)

logging.debug(f"Encoded Bits: {encoded_bits}")

# Viterbi Decoding

decoded_bits = viterbi_decoder(encoded_bits)

logging.debug(f"Decoded Bits: {decoded_bits}")

# Compare

assert decoded_bits is not None, "Viterbi decoding failed."

assert np.array_equal(original_bits, decoded_bits[:len(original_bits)]), "Decoded bits do not match original."

print("Convolutional Encoding/Decoding Test Passed.")

except AssertionError as ae:

print(f"Convolutional Encoding/Decoding Test Failed: {ae}")

except Exception as e:

print(f"Convolutional Encoding/Decoding Test Failed: {e}")

- Run the Convolutional Encoding/Decoding Test: Execute test_conv_encoding_decoding() to ensure that convolutional encoding and Viterbi decoding work correctly.

Expected Output:

Copy code

Convolutional Encoding/Decoding Test Passed.

Interpretation:

  - If Passed: Convolutional encoding and decoding are correctly implemented.

  - If Failed: There's an issue in how convolutional encoding or Viterbi decoding is handled.

Benefit: Isolating convolutional encoding and decoding helps ensure that this critical part of the pipeline works correctly before integrating it into the full pipeline.

5. Check Pilot Sequence Handling

Issue:

Incorrect handling of pilot sequences can lead to misalignment and data corruption.

Solution:

Ensure that pilot sequences are correctly inserted and removed without altering the actual data.

- Verify Pilot Insertion: Ensure that the insert_pilot_sequence function correctly prepends the pilot symbols.

python

Copy code

def insert_pilot_sequence(symbols, pilot_length=64, pilot_value=7):

"""

Insert a pilot sequence at the beginning of the symbol array.

"""

pilot_sequence = np.full(pilot_length, pilot_value, dtype=np.int32)

return np.concatenate((pilot_sequence, symbols))

- Verify Pilot Removal: Ensure that the exact number of pilot symbols is removed during reception.

python

Copy code

# Remove pilot symbols

symbols_without_pilot = quantized_symbols[pilot_length:]

- Test Pilot Handling: Create a small test to ensure pilot insertion and removal works as expected.

python

Copy code

def test_pilot_handling():

"""

Test pilot insertion and removal.

"""

try:

symbols = np.array([1, 3, 5, 7, -1, -3, -5, -7], dtype=np.int32)

pilot_length = 2

pilot_value = 7

# Insert pilot

symbols_with_pilot = insert_pilot_sequence(symbols, pilot_length, pilot_value)

print("Symbols with Pilot:", symbols_with_pilot)

# Remove pilot

symbols_no_pilot = symbols_with_pilot[pilot_length:]

print("Symbols without Pilot:", symbols_no_pilot)

# Compare

if np.array_equal(symbols, symbols_no_pilot):

print("Pilot Handling Test Passed.")

else:

print("Pilot Handling Test Failed.")

except Exception as e:

print(f"Pilot Handling Test Failed: {e}")

- Run the Pilot Handling Test: Execute test_pilot_handling() to ensure correct pilot insertion and removal.

Expected Output:

less

Copy code

Symbols with Pilot: [7 7  1  3  5  7 -1 -3 -5 -7]

Symbols without Pilot: [1 3 5 7 -1 -3 -5 -7]

Pilot Handling Test Passed.

Benefit: Ensuring correct pilot handling prevents synchronization issues that can corrupt data.

6. Validate Bitstream Alignment and Padding

Issue:

Incorrect padding or bitstream alignment can lead to mismatches between encoding and decoding.

Solution:

Ensure that padding is correctly handled and that bitstreams align as expected.

- Check Padding in convert_bits_to_bytes: Remove unnecessary np.clip since np.packbits ensures byte values are within 0-255.

python

Copy code

def convert_bits_to_bytes(bitstream):

"""

Convert a bitstream to a byte array, padding if necessary.

"""

padding_length = (8 - len(bitstream) % 8) if len(bitstream) % 8 != 0 else 0

if padding_length:

bitstream = np.pad(bitstream, (0, padding_length), 'constant')

logging.debug(f"convert_bits_to_bytes: Padded bitstream with {padding_length} zeros.")

byte_array = np.packbits(bitstream)

return byte_array  # Removed np.clip as packbits ensures values are within 0-255

- Ensure Correct Bitstream Lengths: After each step, log the bitstream length to verify alignment.

python

Copy code

logging.debug(f"Bitstream length after step X: {len(bitstream)} bits.")

- Adjust Padding Strategy: When mapping bits to symbols and back, ensure that padding bits don't interfere with the original data.

python

Copy code

# In mapping functions, handle padding carefully

if len(bitstream) % 3 != 0:

padding_length = (3 - len(bitstream) % 3) % 3

bitstream = np.pad(bitstream, (0, padding_length), 'constant')

logging.debug(f"Padded bitstream with {padding_length} zeros for 8-VSB mapping.")

- Test Bitstream Conversion: Create a test to ensure that converting bits to bytes and back preserves the data.

python

Copy code

def test_bitstream_conversion():

"""

Test converting bits to bytes and back.

"""

try:

original_bits = np.random.randint(0, 2, 16, dtype=np.uint8)  # 16 bits

print("Original Bits:", original_bits)

bytes_converted = convert_bits_to_bytes(original_bits)

print("Converted Bytes:", bytes_converted)

bits_recovered = convert_bytes_to_bits(bytes_converted)

print("Recovered Bits:", bits_recovered[:16])  # Only the original bits

if np.array_equal(original_bits, bits_recovered[:16]):

print("Bitstream Conversion Test Passed.")

else:

print("Bitstream Conversion Test Failed.")

except Exception as e:

print(f"Bitstream Conversion Test Failed: {e}")

- Run the Bitstream Conversion Test: Execute test_bitstream_conversion() to ensure accurate conversion.

Expected Output:

less

Copy code

Original Bits: [1 0 1 1 0 1 0 1 1 1 0 0 1 1 1 0]

Converted Bytes: [181 206]

Recovered Bits: [1 0 1 1 0 1 0 1 1 1 0 0 1 1 1 0]

Bitstream Conversion Test Passed.

Benefit: Ensuring correct bitstream alignment and padding prevents data corruption during encoding and decoding.

7. Ensure Symmetric Modulation and Demodulation

Issue:

Mismatch between modulation and demodulation processes can introduce data corruption.

Solution:

Ensure that modulation and demodulation are correctly implemented and are inverses of each other.

- Check Symbol Mapping and Demapping: Since your test_symbol_mapping passed, ensure similar logic applies in the full pipeline.

- Validate Modulation and Demodulation: Ensure that modulation and demodulation processes correctly handle the symbol values.

- Test Modulation and Demodulation Independently: Create a test function to ensure that modulating and demodulating symbols recover the original symbols.

python

Copy code

def test_modulation_demodulation():

"""

Test modulation and demodulation to ensure symbols are correctly recovered.

"""

try:

# Sample symbols

symbols = np.array([-7, -5, -3, -1, 1, 3, 5, 7], dtype=np.int32)

logging.debug(f"Original Symbols: {symbols}")

# Insert pilot

symbols_with_pilot = insert_pilot_sequence(symbols, pilot_length=2)

logging.debug(f"Symbols with Pilot: {symbols_with_pilot}")

# Modulate

modulated = generate_modulated_signal(symbols_with_pilot, CARRIER_FREQ, SAMPLING_RATE)

logging.debug(f"Modulated Signal Length: {len(modulated)} samples.")

# Demodulate

demodulated = demodulate_8vsb(modulated, CARRIER_FREQ, SAMPLING_RATE, LOWPASS_FILTER)

logging.debug(f"Demodulated Signal Length: {len(demodulated)} samples.")

# Timing Recovery

expected_length = len(symbols_with_pilot)

timing_corrected = timing_recovery(demodulated, SPS, expected_length)

logging.debug(f"Timing Corrected Signal Length: {len(timing_corrected)} samples.")

# Quantize Symbols

quantized = quantize_symbols(timing_corrected, VSB_LEVELS)

logging.debug(f"Quantized Symbols: {quantized}")

# Remove Pilot

symbols_no_pilot = quantized[2:]

logging.debug(f"Symbols without Pilot: {symbols_no_pilot}")

# Compare

if np.array_equal(symbols, symbols_no_pilot):

print("Modulation/Demodulation Test Passed.")

else:

print("Modulation/Demodulation Test Failed.")

print(f"Recovered Symbols: {symbols_no_pilot}")

except Exception as e:

print(f"Modulation/Demodulation Test Failed: {e}")

- Run the Modulation/Demodulation Test: Execute test_modulation_demodulation() to ensure accurate symbol recovery.

Expected Output:

Copy code

Modulation/Demodulation Test Passed.

Interpretation:

  - If Passed: Modulation and demodulation processes are correctly implemented.

  - If Failed: There's an issue in how symbols are handled during modulation or demodulation.

Benefit: Ensures that the modulation and demodulation steps are correctly implemented and do not introduce data corruption.

8. Comprehensive Step-by-Step Validation

Issue:

Errors can be introduced at any step of the pipeline, making it challenging to identify the exact point of failure.

Solution:

Validate each step of the pipeline independently to ensure data integrity throughout the process.

Steps:

- Run Simplified Tests: Execute the following test functions to ensure each part works correctly:

  - test_rs_codec()

  - test_symbol_mapping()

  - test_bitstream_conversion()

  - test_conv_encoding_decoding()

  - test_pilot_handling()

  - test_modulation_demodulation()

Benefit: Confirms that each individual component functions as expected before integrating them into the full pipeline.

- Monitor Bitstream Integrity: In your test_pipeline_simplified, add assertions or logging to verify that the bitstream remains unchanged through each step.

python

Copy code

assert np.array_equal(test_bits, rs_decoded_trimmed), "Bitstream mismatch after decoding."

- Check Bitstream Lengths: Ensure that after each step, the bitstream length aligns with expectations.

python

Copy code

logging.debug(f"Bitstream length after step X: {len(bitstream)} bits.")

- Use Controlled Data: Instead of random bits, use a known pattern to make it easier to trace and verify.

python

Copy code

original_bits = np.array([0, 1, 0, 1, 1, 0, 1, 0, 1, 1], dtype=np.int8)

- Handle Padding Carefully: Ensure that any padding introduced does not interfere with the original data.

python

Copy code

# In mapping functions, handle padding carefully

if len(bitstream) % 3 != 0:

padding_length = (3 - len(bitstream) % 3) % 3

bitstream = np.pad(bitstream, (0, padding_length), 'constant')

logging.debug(f"Padded bitstream with {padding_length} zeros for 8-VSB mapping.")

- Run the Full Simplified Pipeline: After ensuring each component works independently, run the simplified pipeline without scrambling and interleaving.

python

Copy code

test_pipeline_simplified()

Expected Outcome:

sql

Copy code

Simplified Test Pipeline Passed: Decoded bitstream matches original.

Interpretation:

  - If Passed: The issue lies in scrambling and interleaving.

  - If Failed: The issue is within convolutional encoding/decoding or another step.

Benefit: Systematically validating each step helps identify the exact point where the data becomes corrupted, making it easier to implement targeted fixes.